

FERRIC NITRATE

Effective Date: 03/06/00


1. Product Identification

Synonyms: Nitric acid, iron (3+) salt nonahydrate; iron nitrate nonahydrate; iron trinitrate 
CAS No.: 10421-48-4 (Anhydrous) 7782-61-8 (Nonahydrate) 
Molecular Weight: 404.00 
Chemical Formula: Fe(NO3)3·9H2O 
Company Identification:
Allan Chemical Corporation





P.O. Box 1837





Fort Lee, NJ 07024

For information call: 1-201-592-8122

For emergencies call: CHEMTEL: 800-255-3924 



2. Composition/Information on Ingredients

  Ingredient                                CAS No         Percent   Hazardous                                       

  ---------------------------------------   ------------   -------   ---------        

  Ferric Nitrate                            10421-48-4     90 - 100%    Yes                                                                  



3. Hazards Identification

Emergency Overview 
-------------------------- 
DANGER! OXIDIZER. CONTACT WITH OTHER MATERIAL MAY CAUSE FIRE. HARMFUL IF SWALLOWED OR INHALED. CAUSES IRRITATION TO SKIN, EYES AND RESPIRATORY TRACT. AFFECTS THE LIVER. 
----------------------------------------------------------------------------------------------------------- 
Potential Health Effects 
---------------------------------- 

Inhalation: 
Causes irritation to the respiratory tract. Symptoms may include coughing, shortness of breath. 
Ingestion: 
Causes irritation to the gastrointestinal tract. Repeated large doses can cause excess iron buildup in the body. Symptoms include gastrointestinal irritation, with abdominal cramps, vomiting, diarrhea, black stool and liver damage. 
Skin Contact: 
Causes irritation to skin. Symptoms include redness, itching, and pain. 
Eye Contact: 
Causes irritation, redness, and pain. Prolonged contact may cause a brownish discoloration to the eye. 
Chronic Exposure: 
Under some circumstances methemoglobinemia occurs in individuals when the nitrate is converted by bacteria in the stomach to nitrite. Chronic exposure may cause liver effects. 
Aggravation of Pre-existing Conditions: 
Persons with pre-existing skin disorders or eye problems, or impaired liver, kidney or respiratory function may be more susceptible to the effects of the substance. 



4. First Aid Measures

Inhalation: 
Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention. 
Ingestion: 
Induce vomiting immediately as directed by medical personnel. Never give anything by mouth to an unconscious person. Get medical attention. 
Skin Contact: 
Immediately flush skin with plenty of water for at least 15 minutes. Remove contaminated clothing and shoes. Get medical attention. Wash clothing before reuse. Thoroughly clean shoes before reuse. 
Eye Contact: 
Immediately flush eyes with plenty of water for at least 15 minutes, lifting lower and upper eyelids occasionally. Get medical attention immediately. 



5. Fire Fighting Measures

Fire: 
Not combustible, but substance is a strong oxidizer and its heat of reaction with reducing agents or combustibles may cause ignition. Fire may produce poisonous or irritating gases. 
Explosion: 
Contact with oxidizable substances may cause extremely violent combustion. Sealed containers may rupture when heated. Sensitive to mechanical impact. 
Fire Extinguishing Media: 
Dry chemical, foam, carbon dioxide, or water spray. 
Special Information: 
Wear full protective clothing and breathing equipment for high-intensity fire or potential explosion conditions. 



6. Accidental Release Measures

Ventilate area of leak or spill. Wear appropriate personal protective equipment as specified in Section 8. Spills: Sweep up and containerize for reclamation or disposal. Vacuuming or wet sweeping may be used to avoid dust dispersal. US Regulations (CERCLA) require reporting spills and releases to soil, water and air in excess of reportable quantities. The toll free number for the US Coast Guard National Response Center is (800) 424-8802. 



7. Handling and Storage

Keep in a tightly closed container. Protect from physical damage. Store in a cool, dry, ventilated area away from sources of heat, moisture and incompatibilities. Separate from combustibles, organic or other readily oxidizable materials. Avoid storage on wood floors. Containers of this material may be hazardous when empty since they retain product residues (dust, solids); observe all warnings and precautions listed for the product. 



8. Exposure Controls/Personal Protection

Airborne Exposure Limits: 
-ACGIH Threshold Limit Value (TLV):
1 mg/m3 (TWA) soluble iron salt as Fe 
Ventilation System: 
A system of local and/or general exhaust is recommended to keep employee exposures below the Airborne Exposure Limits. Local exhaust ventilation is generally preferred because it can control the emissions of the contaminant at its source, preventing dispersion of it into the general work area. Please refer to the ACGIH document, Industrial Ventilation, A Manual of Recommended Practices, most recent edition, for details. 
Personal Respirators (NIOSH Approved): 
If the exposure limit is exceeded, a full facepiece respirator with dust/mist filter may be worn up to 50 times the exposure limit or the maximum use concentration specified by the appropriate regulatory agency or respirator supplier, whichever is lowest. For emergencies or instances where the exposure levels are not known, use a full-facepiece positive-pressure, air-supplied respirator. WARNING: Air purifying respirators do not protect workers in oxygen-deficient atmospheres. 
Skin Protection: 
Wear protective gloves and clean body-covering clothing. 
Eye Protection: 
Use chemical safety goggles. Maintain eye wash fountain and quick-drench facilities in work area. 



9. Physical and Chemical Properties

Appearance: Violet crystals. 
Odor: Slight nitric acid odor. 
Solubility: Freely soluble. 
Specific Gravity: 1.684 
pH: No information found. 
% Volatiles by volume @ 21( C (70( F): 0 
Boiling Point: < 100( C (< 212( F) Decomposes. 
Melting Point: 47.2( C (117( F) 
Vapor Density (Air=1): No information found. 
Vapor Pressure (mm Hg): No information found. 
Evaporation Rate (BuAc=1): No information found. 



10. Stability and Reactivity

Stability: 
Stable under ordinary conditions of use and storage. 
Hazardous Decomposition Products: 
Emits nitrous oxides when heated to decomposition. 
Hazardous Polymerization: 
Will not occur. 
Incompatibilities: 
Substance may react violently with some organic compounds or reducing agents. 
Conditions to Avoid: 
Heat, shock, friction, incompatibles. 



11. Toxicological Information


Oral rat LD50: 3250 mg/kg 

  --------\Cancer Lists\------------------------------------------------------

                                         ---NTP Carcinogen---

  Ingredient                             Known    Anticipated    IARC Category

  ------------------------------------   -----    -----------    -------------

  Ferric Nitrate (10421-48-4)             No          No            None



12. Ecological Information

Environmental Fate: 
No information found. 
Environmental Toxicity: 
No information found. 



13. Disposal Considerations

Whatever cannot be saved for recovery or recycling should be handled as hazardous waste and sent to a RCRA approved waste facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements. 



14. Transport Information

Domestic (Land, D.O.T.) 
----------------------- 
Proper Shipping Name: FERRIC NITRATE 
Hazard Class: 5.1 
UN/NA: UN1466 
Packing Group: III 
 
International (Water, I.M.O.) 
----------------------------- 
Proper Shipping Name: FERRIC NITRATE 
Hazard Class: 5.1 
UN/NA: UN1466 
Packing Group: III 



15. Regulatory Information

  --------\Chemical Inventory Status - Part 1\---------------------------------

  Ingredient                                       TSCA  EC   Japan  Australia

  -----------------------------------------------  ----  ---  -----  ---------

  Ferric Nitrate (10421-48-4)                       Yes  Yes   Yes      Yes                                      

  --------\Chemical Inventory Status - Part 2\---------------------------------

                                                          --Canada--

  Ingredient                                       Korea  DSL   NDSL  Phil.

  -----------------------------------------------  -----  ---   ----  -----

  Ferric Nitrate (10421-48-4)                       Yes   Yes   No     Yes                             

  --------\Federal, State & International Regulations - Part 1\----------------

                                             -SARA 302-    ------SARA 313------

  Ingredient                                 RQ    TPQ     List  Chemical Catg.

  -----------------------------------------  ---   -----   ----  --------------

  Ferric Nitrate (10421-48-4)                No    No      No    Nitrate Cmpd

  --------\Federal, State & International Regulations - Part 2\----------------

                                                        -RCRA-    -TSCA-

  Ingredient                                 CERCLA     261.33     8(d) 

  -----------------------------------------  ------     ------    ------

  Ferric Nitrate (10421-48-4)                1000       No         No                   

Chemical Weapons Convention:  No     TSCA 12(b):  No     CDTA:  No

SARA 311/312:  Acute: Yes      Chronic: Yes  Fire: Yes Pressure: No

Reactivity: Yes         (Mixture / Solid)



Australian Hazchem Code: 1[T] 
Poison Schedule: None allocated. 
WHMIS: 
This MSDS has been prepared according to the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all of the information required by the CPR. 



16. Other Information

NFPA Ratings: Health: 1 Flammability: 0 Reactivity: 1 Other: Oxidizer 
Product Use: 
Laboratory Reagent. 
Revision Information: 
MSDS Section(s) changed since last revision of document include: 1, 3, 16. 
Disclaimer: 
****************************************************************************** 
Allan Chemical Corporation provides the information contained herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. ALLAN CHEMICAL CORPORATION MAKES NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT TO THE INFORMATION SET FORTH HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS. ACCORDINGLY, ALLAN CHEMICAL CORPORATION WILL NOT BE RESPONSIBLE FOR DAMAGES RESULTING FROM USE OF OR RELIANCE UPON THIS INFORMATION. 
****************************************************************************** 
 

